# ASSUMPTIONS DOCUMENT - AI-BASED TRAFFIC SIGNAL CONTROLLER USING VLSI

## Platform and tools

- Language: SystemVerilog (IEEE 1800);

- Simulators: ModelSim Intel FPGA Edition 2020.1

- Single clock domain clk; testbench clock period 10 ns. Active-high synchronous reset on every register.

- The design is simulated only (no synthesis/timing closure in this project scope).

## Timing conventions

- Every phase lasts exactly its configured number of clock cycles (e.g. GREEN_TIME=3 -> 3 cycles).

- generic_timer: start loads the target; done is a one-cycle pulse (target+1) cycles after the start request; count_target=0 asserts done the cycle after start; reset clears everything. The controller compensates with TIMER_LATENCY.

- All-Red is non-zero (RED_TIME >= 1, enforced by a parameter check) between every pair of conflicting directions.

## Pedestrian handling

- A pedestrian request is the rising edge of ped_req; holding the button high is one request.

- Requests are latched and serviced only at an All-Red boundary, and only when the arbiter grants; one request gives at most one pedestrian phase.

- A request arriving in the same cycle a pedestrian phase begins counts as a new request.

- Arbitration: round-robin with a held grant; reset preference is A; the preference pointer moves only on contested decisions, so contested winners alternate.

- A pedestrian phase in progress is aborted by an emergency and its request is not re-queued.

## Emergency

- A single emergency_override input; both junctions reach All-Red one clock after the override is sampled (requirement: <= 2).

- While asserted, both stay All-Red. After release a full RED_TIME All-Red interval completes, then the junctions restart through NS_GREEN under the green-wave gate (a valid safe phase, never a partial phase).

- Emergency overrides the normal FSM; it does not use reset, so it never conflicts with the reset policy.

## Green wave

- Junction A is the reference. Offset = clock cycles from A's first NS-green cycle to B's first NS-green cycle; target 4, accepted window 3..5; measured value is exactly 4 in every simulation.

- B may idle in All-Red (NS_WAIT) until released; A waits for B to be parked before starting its next NS-green, so the offset is independent of green/pedestrian durations.

## Adaptive timing

- traffic_density is 3 bits (0..7). Green = clamp(GREEN_TIME + density x GREEN_EXT_STEP, MIN_GREEN_TIME, MAX_GREEN_TIME).

- Density is sampled once, when the green phase is requested; changes during a green are ignored until the next green phase.

- Parameter values used in verification: A: base 4, step 2, min 3, max 12; B: base 2 (below its floor on purpose, so the clamp is exercised), step 1, min 3, max 8 (system verification bench; amtgc_top defaults use base 3); standalone controller testbench uses different values on purpose to exercise floor and cap.

- Traffic density is an abstract input; no sensor model is implemented.

## Verification scope

- Self-checking by monitors/scoreboards that stop the simulation with $fatal on the first violation.

- Random run length and justification: see tb_amtgc_full.sv header (20000 cycles).

- Coverage statement compares the Task 1 plan (T01..T20) with executed tests; any uncovered item is listed with a reason.