# BUG LOG - AI-Based Traffic Signal Controller using VLSI

Honest record of defects found during design and verification, the earlier decision each one traces back to, and the fix. Status "reviewed" means the bug was found by design review / hand-tracing BEFORE the first simulation of that code; "simulation" means a failing self-checking test exposed it.

| ID | Found in | How found | Traces back to | Description | Fix | Verified by |
| --- | --- | --- | --- | --- | --- | --- |
| BUG-01 | Task 2 review | Reviewed (hand-trace) | Task 1 sec. 4.5 / 13 (green-wave policy) | Plan had a "coordination logic" box but the controller had no hook to delay B. Junctions with different cycle lengths drift apart, so a constant offset is impossible; after an emergency both would restart together (offset 0). | Added ns_go input, ns_wait output and NS_WAIT state to junction_controller; B released by a generic_timer offset started from A's NS-green; A waits for B parked. | tb_amtgc_top / tb_amtgc_full: offset measured on every B start (292 in the system bench, 835 in the unified bench, always 4) |
| BUG-02 | Task 2 RTL | Reviewed (hand-trace of timer/FSM timing) | Task 2 sec. 4 (timer semantics) | done appears (target+1) cycles after the start request, so every FSM phase would last TIME+1 cycles instead of TIME. | TIMER_LATENCY localparam; controller loads TIME - TIMER_LATENCY. | run_len_checker asserts exact phase lengths on every run |
| BUG-03 | Task 2 RTL | Reviewed | Task 2 sec. 6 (emergency recovery flag) | If a stale timer done coincided with the emergency-release cycle, the recovery logic jumped to NS_GREEN while the timer had just been loaded with RED_TIME, giving a green of RED_TIME length. | Timer restarted every cycle while override is high; recovery flag removed; All-Red interval always completes after release. | Repeated emergency toggle (no idle cycle) + 24-offset sweep + random emergency |
| BUG-04 | Task 2 testbenches | Reviewed | Task 2 sec. 8/9 | (a) ped_count incremented by the testbench itself, so the "serviced once" check could never fail; (b) grant tied high; (c) "three parameter values" were three targets with the same COUNTER_WIDTH; (d) timer test lacked early-done, pulse-width and no-spurious-done checks. | Monitor counts real pedestrian-phase entries; TB drives the grant; widths 4/8/16; extra checks added. | tb_generic_timer, tb_junction_controller |
| BUG-05 | Task 3 arbiter | Reviewed while writing fairness test | Task 3 fairness requirement | Round-robin pointer was moved on every grant, including uncontested ones, so contested winners did not alternate (A wins, B served next, pointer says A again). | Pointer moves only on contested decisions. | 60 simultaneous rounds: A first 30, B first 30; TB fails on two equal consecutive winners |
| BUG-06 | Task 3 integration | Reviewed | Task 2 sec. 5 (request policy) | A request visible to the arbiter during the junction's own pedestrian phase (or arriving the cycle the phase starts) would keep the grant held and could starve the other junction. | ped_req_pending output masked during PED_PHASE so the arbiter always sees the request drop. | Fairness rounds + pedestrian starvation watchdog (1500 cycles) |
| PROC-01 | Task 3 bring-up | Simulation (compile) | Process | Compile errors caused by a missing tb/tb_monitors.sv and stale old testbench files, not by RTL. | Copied correct files; old files kept as .bak outside the build. | Clean compile |

## Task 5 Mutation findings

| ID | Test that failed | Failure message | Root cause | Earlier decision | Fix | Re-run result |
| --- | --- | --- | --- | --- | --- | --- |
| BUG-07 | Mutation test 4 (minimum-green clamp removed) | Mutant SURVIVED: full bench still printed ALL PASS (B density-1 green = 4) | Verification gap, not an RTL bug: in tb_amtgc_full junction B's base green (3) equalled its floor (3), so the clamp never changed any value | Task 5 bench parameter choice (Task 4 test plan T18) | B base green set to 2 (below floor 3) in tb_amtgc_full; expectation model already computes the clamp | Re-run: mutant now FAILS with [B] NS_GREEN duration FAIL: expected 3 cycles, observed 2 at 145 ns; restored RTL passes (835 green-wave measurements, ALL PASS) |

In Task 5 no additional defects were exposed by the unified bench