# AMTGC - AS-BUILT DOCUMENTATION, TASKS 1-4

Diagrams: docs/diagrams/task1_asbuilt_block_diagram.svg, task2_asbuilt_fsm_diagram.svg (plan version: task1_planned_block_diagram.svg).

Evidence: sim/Screenshots/, sim/logs/ (names in SCREENSHOT_MANIFEST.md).

Base: the training project (traffic_timer, traffic_fsm_final, traffic_system_top_final), extended - not replaced.

## Task 1 - Planning and architecture

Architecture. amtgc_top holds two junction_controller instances (A reference, B follower), one ped_arbiter, and a wave timer (generic_timer). One clock, active-high synchronous reset, named ports only.

## Task 2 - Parameterized timer and junction FSM

- generic_timer: COUNTER_WIDTH, start, count_target, one-cycle done; target 0 completes on the start edge; latched target. done arrives target+1 cycles after the start request, so the controller loads TIME-1 (TIMER_LATENCY=1) and every phase lasts exactly TIME cycles.

- junction_controller: Moore FSM, no counter inside, parameters for all times, default branch to All-Red, sanity $fatal checks.

- Verification: tb_generic_timer at widths 4/8/16 (target n, 0, 1, reset-while-counting) - all PASS. tb_junction_controller - normal sequence, min-green floor, density sweep, ped during yellow, emergency sweep, emergency during ped, repeated toggle, reset - ALL PASS.

- Evidence: Task2_01..03 screenshots, task2_timer_and_junction_console.log.

## Task 3 - Integration

- amtgc_top wires both controllers, arbiter and wave logic. Pedestrian request path: controller ped_req_pending -> arbiter -> ped_grant; pending is masked during the ped phase so the arbiter sees the request drop.

- Results (final bench): green-wave offset 4 cycles in all 292 measurements (window 3..5); 60 simultaneous ped rounds, A first 30 / B first 30; emergency during NS/EW green and ped phase; emergency sweep over 24 phase offsets; reset during operation - ALL PASS.

- Evidence: Task3_01, Task3_02, Task3_03 (offset, cursors 75000/115000 ps, delta 40000 ps), task3_integration.log.

## Task 4 - Adaptive timing

- green = clamp(GREEN_TIME + density x GREEN_EXT_STEP, MIN_GREEN_TIME, MAX_GREEN_TIME), 3-bit density, sampled once at green start.

- Controller bench (A: 3/1/3/6, B: 2/2/4/9): A green 3,4,5,6,6,6,6,6; B green 4,4,6,8,9,9,9,9 for density 0..7. Density change during green is ignored until the next green.

- System bench: both junctions at max density, A green=12, B green=8, NS and EW both still served (no starvation); green wave stays at 4 cycles under 8 density combinations.

- Written answers

  - Was the timer modified? No - generic_timer.sv is byte-identical to Task 2.

  - When is density sampled? Once, at green entry, so a mid-green change cannot create an unsafe or truncated phase.

  - Why in the controller, not the timer? The timer stays a reusable counter; the policy (clamp, step) belongs to the FSM that owns the phase.

  - Max density starvation? Max green is capped, so the other direction and pedestrians are always reached (watchdog 1500 cycles never fired).

- Evidence: Task4_01..04, task4_density.log.

## Notes

- File and screenshot names follow the repository layout in README.md and SCREENSHOT_MANIFEST.md.

- RTL integrity: RTL_CHECKSUMS.txt lists the SHA-256 of the RTL that passed the final Task 5 run.