# TASK 5 - COVERAGE STATEMENT

(TASK 1 PLAN VS. TESTS EXECUTED)

Evidence: sim/logs/task5_full_verification.log (Run to completion at 242175 ns, AMTGC FULL VERIFICATION: ALL PASS, 0 compile errors/warnings).

Mutation runs: sim/logs/task5_mutation_tests.log.

Bench: tb/tb_amtgc_full.sv with tb/tb_monitors.sv. Checks are self-checking ($fatal on first violation); waveforms are supporting evidence only.

## Plan items T01-T20

| ID | Scenario (Task 1 plan) | Executed by | Log evidence | Status |
| --- | --- | --- | --- | --- |
| T01 | Reset at startup | Startup reset; monitors require All-Red one clock after reset and NS restart | [sim time 965 ns] T17 line | Covered |
| T02 | Normal NS sequence | Free-running operation; legal-step/phase-order monitor | same | Covered |
| T03 | Normal EW sequence | same | same | Covered |
| T04 | All-Red safety gap | Monitor fails on any conflicting green without All-Red | same (and all later runs) | Covered |
| T05 | Timer target = 0 | timer_test_unit x3 widths | TIMER TEST n PASS ... target=0 | Covered |
| T06 | Timer reset while counting | timer_test_unit x3 widths | ... reset-while-counting | Covered |
| T07 | Pedestrian A | Single request | [1135 ns] T07/T08 | Covered |
| T08 | Pedestrian B | Single request | [1135 ns] | Covered |
| T09 | Simultaneous requests | Round-robin, first contested winner = A | [15125 ns] T09/T10 | Covered |
| T10 | Repeated simultaneous requests | 60 rounds = 120 requests, winners alternate (A first 30, B first 30) | [15125 ns] | Covered |
| T11 | Request during yellow | Retained, serviced exactly once | [17335 ns] | Covered |
| T12 | Emergency in NS green | Directed | [17535 ns] | Covered |
| T13 | Emergency in EW green | Directed | [17705 ns] | Covered |
| T14 | Emergency in pedestrian phase | Directed (A and B) | [18105 ns] | Covered |
| T15 | Emergency held | 10-cycle hold, both stay All-Red | [17535 ns] | Covered |
| T16 | Emergency release | Safe restart through NS_GREEN checked by monitor | [17535 ns] | Covered |
| T17 | Green-wave offset | Measured on every B NS-green start: 835 measurements, offset 4..4 (window 3..5) | SUMMARY line | Covered |
| T18 | Minimum density | Sweep; B base 2 is clamped up to its floor 3 in the system bench (standalone bench also tests floor 4) | [32695 ns] | Covered |
| T19 | Maximum density | Cap reached (A 12, B 8) and never exceeded | [32695 ns], [33885 ns] | Covered |
| T20 | Density sweep | All 8 values, both junctions | [32695 ns] table | Covered |

## Additional Task 5 / corner-case tests

| ID | Test | Log evidence |
| --- | --- | --- |
| X21 | Emergency sweep over 24 phase offsets | [29605 ns] |
| X22 | Repeated emergency assert/deassert with no idle cycle | [29925 ns] |
| X23 | Reset during active operation | [30445 ns] |
| X24 | Both junctions at maximum density, neither direction starved | [33885 ns] |
| X25 | Density change during green ignored (sampled at green start) | [34455 ns] |
| X26 | Green wave under 8 density combinations | [41505 ns] |
| X27 | Constrained-random, 20000 cycles (density every cycle, pedestrian ~1/20, emergency ~1/150, reset ~1/3000) | [242175 ns] |
| X28 | Emergency hit every phase of both junctions (12 bins) | coverage table |
| X29 | Pedestrian starvation watchdog (1500 cycles) never fired | coverage table |

Totals from the run: 835 green-wave measurements (all 4 cycles); pedestrian phases served A = 684, B = 657. An additional directed test ([20125 ns]) injects an emergency in each phase (all-red, NS G/Y, EW G/Y, pedestrian) for both junctions.

## Not covered / limitations

- No tool-based coverage metrics. ModelSim Intel FPGA Edition was used without code/functional coverage collection; coverage is scenario-level plus the emergency-phase bins implemented in the testbench.

- Single random seed in the delivered log. (Optional strengthening: rerun with other seeds.)

- All-Red and NS_WAIT share one emergency bin, because both show identical lights; they are not distinguished.

- Procedural monitors instead of SVA properties; equivalent checks, but no assertion-coverage report.

- Emergency during a pedestrian phase aborts that service and does not re-queue it (documented assumption).

- Emergency response is checked at 1 clock after the override is sampled (stricter than the 2-cycle requirement).

- Simulation only: no synthesis, gate-level or formal verification.

## Negative (mutation) tests

Each mutant was applied to the RTL, the full bench (tb_amtgc_full) was run, and the RTL was restored afterwards.

Log: sim/logs/task5_mutation_tests.log: mutation 1 = lines 4-46, mutation 2 = lines 47-107, mutation 3 = lines 108-164, mutation 4 = lines 165-207, restored clean run = lines 208-335.

| # | Mutation (file) | Expected | Observed Fatal line | Sim time | Result |
| --- | --- | --- | --- | --- | --- |
| 1 | All-Red gap skipped (junction_controller.sv) | safety monitor fires | [A] SAFETY: conflicting green without All-Red gap | 145 ns | CAUGHT |
| 2 | Emergency override disabled (junction_controller.sv) | emergency check fires | [A] EMERGENCY/RESET: not All-Red one clock after assertion | 17365 ns | CAUGHT |
| 3 | Fixed-priority arbiter (ped_arbiter.sv) | fairness check fires | ARBITER FAIRNESS FAIL: same junction won contested rounds 0 and 1 | 1515 ns | CAUGHT |
| 4 | Minimum-green clamp removed (junction_controller.sv) | phase-length check fires | [B] NS_GREEN duration FAIL: expected 3 cycles, observed 2 | 145 ns | CAUGHT (after bench fix BUG-07) |

Restored RTL, clean re-run: 29/29 requirements COVERED, green-wave measurements = 835 (offset 4..4), A ped = 684, B ped = 657, AMTGC FULL VERIFICATION: ALL PASS.

Conclusion: all four injected defects are detected, so the checks are capable of failing. Mutation 4 first survived (BUG-07); it was caught once junction B's base green was set below its floor.